- U-easytech                       STM3220V-CRE/UET

- STM3220V-CRE/UET BOARD

Description:

The STM3220V-CRE/UET board has been designed as a complete demonstration and development platform for the ARM cortex-M3 core-based STM32F207VGT6 microcontroller. It features three I2Cs, three SPIs, six USARTs, two CANs, three 12-bit ADCs, two 12-bit DACs, internal 128-KByte Data SRAM and 1024-KByte Flash, USB FS, JTAG debugging support. This evaluation board can be used as the reference design for user application development but it is not considered as a final application. With the JTAG interface you can use debugger STLINKIII or JLINKV8 to test your program.

- Features:

- MCU: STM32F207VGT6 (100pin)

- Core work at up to 120M frequency

- 1024KB Flash and 128KB RAM

- USB Device、SDIO interface

- l 2.8’ TFT LCD

- 1 CAN channel port

- 4 User Leds, 6 User Keys

- 1 Power Key, 1 Reset Key

- 1 Power Led

- 1 DV 5V power interface

- 1 I2C EEPROM 24C02

- 1 SPI FLASH AT45DB161

- 1 AD channel with Pointer

- l RTC with backup battery interface

- 1 camera OV9655 module

- USB OTG FS interface

- l JTAG Debug port

- l ALL I/O expansion

Order NO: STM3220V-CRE/UET

System description：

4 User Ledss

Reset Key

6 User Keys

TFT LCD

& touch

SPI or SDIO interface

SPI Flash

AT45DB/M25P

I2S interface

Boot SW Jump

DC 5V IN

Camera for F2/F4 CPU

Ethernet for F2/F4 CPU

ADC Pointer

CAN chip

CAN interface

24L01 CN

DIP/SOP

MC control

interface

nRF24L01

interface

Part GPIOs

Back-up

Battery

STM32F207VGT6

CORE BOARD

Power Led

JTAG  CN

Two kinds

Interface

Of USB

Power Key

- U-easytech                       STM3220V-CRE/UET

- STM3220V-CRE/UET 开发板

简介:

STM3220V-CRE/UET开发板是一块基于CORTEX-M3内核的STM32F207VGT6微处理器的开发平台。STM32F207VGT6 有3个I2C,3个SPI,6个USART,2路CAN,3个12位ADC转换器，2个12位DAC转换器，内部有128KB的Data SRAM，1024KB的FLASH,支持USB FS和JTAG调试。此评估平台可作为用户应用开发时做参考，而未作最终产品应用考虑。用户可通过JTAG用STLINK或JLINK仿真下载程序。

- 特点:

- CPU:STM32F207VGT6 (100pin)

- 最大工作频率：120M

- 1024K FLASH,128K RAM

- 具有USB 从设备接口和SDIO接口

- 一个2.8 TFT 液晶

- 一路CAN通道接口

- 4个用户指示灯，6个用户按键

- 1个电源开关，一个复位键

- 一个电源指示灯

- 一个DV5V输入接口

- 一个24C02 存储芯片

- 一个SPI FLASH  AT45DB161

- 一个AD电位器

- 电池接口

- 摄像头模块接口

- 以太网模块接口

- 一个JTAG口

- 所有IO引出

订单编号: STM3220V-CRE/UET